Reconstructing 3D nanoarchitecture of solid-liquid interface – from atomic scale electrical double layer to solid-electrolyte interphase

Oleg V. Kolosov and Yue Chen

Department of Physics, Lancaster University, Lancaster, LA1 4YB, United Kingdom

The Faraday Institution, Quad One, Harwell Science and Innovation Campus, OX11 0RA, Didcot, United Kingdom

o.kolosov@lancaster.ac.uk

We use advanced scanning probe microscopy to explore one of the most enigmatic and, arguably, most important parts of rechargeable batteries – the solid electrolyte interphase (SEI) layer, its dynamics and, significantly, its nanoscale structure and composition that hold clues to high-performing and safe energy storage. We linked electrochemical AFM and a recently developed 3D nano-rheology scanning probe microscopy to study in-situ the dynamic formation of SEI starting from a few Å thick electrical double-layer (EDL) to the full 3D nanostructure of SEI. Nanoscale rheology maps allow to quantify in real-time 3D distribution of various components in the as formed SEI layer, revealing the dominant role of a solvent-guided molecular-scale EDL structure. Instigated by energy storage applications, this study establishes a new generic approach allowing to explore the nanoscale internal structure of soft materials via 3D mapping of their local nanorheological properties – with potential applications ranging from biological objects to artificial biomaterials to industrial coatings.

Yue Chen and Oleg, V. Kolosov, Exploring the ‘gel’ part of solid-electrolyte-interphase in rechargeable batteries – via “3D nanorheology microscopy, Microscopy and analysis, Wiley, March/April 2022, p.10 https://analyticalscience.wiley.com/do/10.1002/micro.2022.59